31/07/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

CASE 1:

4 Relay Module: Serial-Controlled Relay LEDs

DESCRIPTION:

This project demonstrates how to control a 4-channel relay module using an Arduino UNO and the Serial Monitor. The relay module has 4 built-in indicator LEDs (one for each relay), which visually show the ON/OFF status of each relay.

CONNECTION:

| Relay Module | Arduino Pin |
| --- | --- |
| IN1 | D2 |
| IN2 | D3 |
| IN3 | D4 |
| IN4 | D5 |
| VCC | 5V |
| GND | GND |

PROGRAM:

int relay1 = 2;

int relay2 = 3;

int relay3 = 4;

int relay4 = 5;

void setup() {

Serial.begin(9600);

pinMode(relay1, OUTPUT);

pinMode(relay2, OUTPUT);

pinMode(relay3, OUTPUT);

pinMode(relay4, OUTPUT);

// Turn OFF all relays initially

digitalWrite(relay1, HIGH);

digitalWrite(relay2, HIGH);

digitalWrite(relay3, HIGH);

digitalWrite(relay4, HIGH);

Serial.println("Type 1, 2, 3, 4 to turn ON relays");

Serial.println("Type A, B, C, D to turn OFF relays");

}

void loop() {

if (Serial.available()) {

char command = Serial.read();

switch (command) {

case '1':

digitalWrite(relay1, LOW); // ON

Serial.println("Relay 1 ON");

break;

case '2':

digitalWrite(relay2, LOW);

Serial.println("Relay 2 ON");

break;

case '3':

digitalWrite(relay3, LOW);

Serial.println("Relay 3 ON");

break;

case '4':

digitalWrite(relay4, LOW);

Serial.println("Relay 4 ON");

break;

case 'A':

digitalWrite(relay1, HIGH); // OFF

Serial.println("Relay 1 OFF");

break;

case 'B':

digitalWrite(relay2, HIGH);

Serial.println("Relay 2 OFF");

break;

case 'C':

digitalWrite(relay3, HIGH);

Serial.println("Relay 3 OFF");

break;

case 'D':

digitalWrite(relay4, HIGH);

Serial.println("Relay 4 OFF");

break;

default:

Serial.println("Invalid command");

break;

}

}

}

BOARD SETUP:

OUTPUT:

CASE 2:

4 Relay Module: Relay LED Sequence

DESCRIPTION:

This project demonstrates how to create a sequence pattern using the LED indicators on a 4-channel relay module, controlled by an Arduino UNO. The relays are turned ON and OFF in a timed sequence to create a chasing/blinking LED effect — just like a light show but using relay click sounds and their LEDs.

BOARD SETUP:

PROGRAM:

int relay1 = 2;

int relay2 = 3;

int relay3 = 4;

int relay4 = 5;

void setup() {

pinMode(relay1, OUTPUT);

pinMode(relay2, OUTPUT);

pinMode(relay3, OUTPUT);

pinMode(relay4, OUTPUT);

// Turn OFF all relays (relay OFF = HIGH for most modules)

digitalWrite(relay1, HIGH);

digitalWrite(relay2, HIGH);

digitalWrite(relay3, HIGH);

digitalWrite(relay4, HIGH);

}

void loop() {

// Relay ON = LOW, Relay OFF = HIGH

// Turn ON one by one

digitalWrite(relay1, LOW);

delay(500);

digitalWrite(relay2, LOW);

delay(500);

digitalWrite(relay3, LOW);

delay(500);

digitalWrite(relay4, LOW);

delay(500);

// Turn OFF one by one

digitalWrite(relay1, HIGH);

delay(500);

digitalWrite(relay2, HIGH);

delay(500);

digitalWrite(relay3, HIGH);

delay(500);

digitalWrite(relay4, HIGH);

delay(500);

// Blink all relays quickly

for (int i = 0; i < 3; i++) {

digitalWrite(relay1, LOW);

digitalWrite(relay2, LOW);

digitalWrite(relay3, LOW);

digitalWrite(relay4, LOW);

delay(300);

digitalWrite(relay1, HIGH);

digitalWrite(relay2, HIGH);

digitalWrite(relay3, HIGH);

digitalWrite(relay4, HIGH);

delay(300);

}

delay(1000); // pause before next loop

}

OUTPUT: