04/08/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

Sound-Activated LED Control Using HW-485 Sensor

HW 485:

HW-485 High-Sensitivity Sound Sensor

A microphone-based module designed to detect ambient sound levels. It outputs both analog and digital signals depending on the sound intensity

Purpose:

- Measure ambient noise levels in the environment.

- Often used for sound-activated projects, like gesture-based lighting, clapping switches, or alarm triggers.

Components & Features:

- Microphone element (electret or condenser) to capture sound

- Pre-amplifier circuit to boost microphone output

- Comparator with potentiometer threshold to select digital output (HIGH/LOW)

- Analog output pin provides continuous voltage proportional to sound intensity.

- Digital output pin changes state when sound exceeds a set threshold.

- Onboard LED visually indicates sound detection through blinking or lighting.

DESCRIPTION:

This project uses the HW-485 sound sensor and Arduino to detect loud sounds like claps and automatically turn an LED ON. When no sound is present, the LED remains OFF. The sensor sends a digital signal to the Arduino when it detects sound, and the Arduino controls the LED based on that signal. It's a simple hands-free light control system useful for home automation and DIY electronics learning.

CONNECTION:

| Sound Sensor | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| DO | D2 |

| LED | Arduino |
| --- | --- |
| Anode | D8 |
| Cathode | GND through 220Ω resistor |

BOARD SETUP:

PROGRAM:

int soundPin = 2;  // Digital Output from sound sensor

int ledPin = 8;    // LED pin

void setup() {

pinMode(soundPin, INPUT);

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int soundState = digitalRead(soundPin);

if (soundState == HIGH) {

digitalWrite(ledPin, HIGH);

Serial.println("👂 Sound Detected – LED ON");

} else {

digitalWrite(ledPin, LOW);

Serial.println("🔇 No Sound – LED OFF");

}

delay(100);  // Small delay to avoid flickering

}

OUTPUT: