19/08/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

Hall Effect Sensor Switch Module

DESCRIPTION:

The Hall Effect sensor is an electronic switch that detects the presence of a magnetic field. When a magnet comes close, the sensor changes its output (ON/OFF). It works without touching, so it is often used as a contactless switch.

CONNECTION:

| Module / Part | Pin Name | Connects To (Arduino Pin) |
| --- | --- | --- |
| HW-492 Sensor | VCC | 5V |
|  | GND | GND |
|  | OUT | D2 |
| Relay Module | VCC | 5V |
|  | GND | GND |
|  | IN | D8 |
| Arduino UNO Pins | D2 | Sensor OUT |
|  | D8 | Relay IN |
|  | 5V | Sensor VCC + Relay VCC |
|  | GND | Sensor GND + Relay GND |

BOARD SETUP:

PROGRAM:

int hallSensor = 2;   // HW-492 OUT pin connected to D2

int relayPin   = 8;   // Relay IN pin connected to D8

void setup() {

pinMode(hallSensor, INPUT);   // Hall effect sensor as input

pinMode(relayPin, OUTPUT);    // Relay as output

digitalWrite(relayPin, LOW);  // Keep relay OFF at start

Serial.begin(9600);           // For debugging

}

void loop() {

int sensorState = digitalRead(hallSensor); // Read sensor

if (sensorState == LOW) {

// Magnet detected

digitalWrite(relayPin, HIGH);   // Turn relay ON

Serial.println("Magnet Detected → Relay ON");

} else {

// No magnet

digitalWrite(relayPin, LOW);    // Turn relay OFF

Serial.println("No Magnet → Relay OFF");

}

delay(100); // Small delay for stability

}

OUTPUT: