18/08/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

GY-521

- MPU-6050 chip

  - A 3-axis accelerometer + 3-axis gyroscope

- Built-in Digital Motion Processor (DMP) for motion tracking

- I2C interface (SCL and SDA pins) for communication with Arduino or other microcontrollers

What does it do?

The GY-521 can:

- Measure acceleration (tilt, movement)

- Measure rotation (angular velocity)

- Detect orientation (for balancing robots, drones, etc.)

- Combine data from both sensors using the DMP for better motion tracking

Common Projects Using GY-521:

- Self-balancing robots

- Gesture-controlled devices

- Drones and RC planes

- Fitness trackers or fall detection

- Motion-activated lights or alarms

Arduino Temperature-Controlled Relay with LED

(HW-506)

DESCRIPTION:

This is a standalone Arduino project that reads temperature using the HW-506 DS18B20 module and controls a relay based on a temperature setpoint.

- When the temperature exceeds the setpoint, the relay turns ON (to drive a fan, heater, or light).

- When the temperature drops below the setpoint, the relay turns OFF.

- An LED indicates relay status: ON when the relay is ON, OFF otherwise.

- No Wi-Fi, Bluetooth, or smartphone required — fully offline.

PIN CONNECTIONS:

| Function | Module Pin | Arduino Pin |
| --- | --- | --- |
| HW-506 DQ (data) | DQ | D2 |
| HW-506 VCC | VCC | 5V |
| HW-506 GND | GND | GND |
| Relay IN | IN | D7 |
| Relay VCC | VCC | 5V |
| Relay GND | GND | GND |
| LED anode (+) | – | D8 (with 220 Ω resistor) |
| LED cathode (−) | – | GND |

BOARD SETUP:

PROGRAM:

#include <OneWire.h>

#include <DallasTemperature.h>

// Pins

const int tempPin = 2;     // DS18B20 DQ

const int relayPin = 8;    // Relay control

const int ledPin = 7;      // Status LED

// Temperature control

const float setPoint = 30.0;  // Set temperature in °C

// Setup DS18B20

OneWire oneWire(tempPin);

DallasTemperature sensors(&oneWire);

void setup() {

Serial.begin(9600);

sensors.begin();

pinMode(relayPin, OUTPUT);

pinMode(ledPin, OUTPUT);

digitalWrite(relayPin, LOW);  // Relay OFF initially

digitalWrite(ledPin, LOW);    // LED OFF initially

}

void loop() {

sensors.requestTemperatures();

float tempC = sensors.getTempCByIndex(0);

Serial.print("Temperature: ");

Serial.print(tempC);

Serial.println(" °C");

if (tempC >= setPoint) {

digitalWrite(relayPin, HIGH);  // Turn ON relay

digitalWrite(ledPin, HIGH);    // Turn ON LED

} else {

digitalWrite(relayPin, LOW);   // Turn OFF relay

digitalWrite(ledPin, LOW);     // Turn OFF LED

}

delay(2000); // 2-second interval

}

OUTPUT: