18/08/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

HW-492: HALL EFFECT SENSOR MODULE WITH A3144

The HW-492 is a digital Hall Effect sensor module that detects magnetic fields using the A3144 Hall sensor. It is designed to sense the presence, absence, or change in magnetic fields and output a corresponding digital signal. The module is often used in proximity sensing, speed measurement, and magnetic switch applications.

PIN DESCRIPTION:

| Pin | Name | Function |
| --- | --- | --- |
| 1 | VCC | Power supply (Connect to 3.3V or 5V) |
| 2 | GND | Ground (Connect to GND of your microcontroller) |
| 3 | OUT | Digital Output pin (Goes LOW when magnet is near) |

- The output is HIGH (1) when no magnetic field is detected.

- The output is LOW (0) when a magnet is near the sensor.

PROGRAM:

const int hallPin = 2;  // Digital pin connected to OUT of HW-492

void setup() {

pinMode(hallPin, INPUT);

Serial.begin(9600);   // Start the Serial Monitor

}

void loop() {

int sensorValue = digitalRead(hallPin);

if (sensorValue == LOW) {

Serial.println("🧲 Magnet Detected!");

} else {

Serial.println("No Magnet.");

}

delay(300); // Delay to avoid spamming the monitor

}

BOARD SETUP:

OUTPUT:

HW-504: JOYSTICK SENSOR

| Pin | Function |
| --- | --- |
| GND | Ground |
| VCC | Power (5V/3.3V) |
| VRx | X-axis output |
| VRy | Y-axis output |
| SW | Button switch |

Specification of Analog Joystick:

- Operating Voltage: 3V to 5V DC.

- Output Type: The output in the form of analog voltage

- Potentiometer: It has an internal 10k Ohm potentiometer used in the joystick to measure the position.

- Operating temperature: It operates on the 0 to 70 °C temperature range.

Projects:

- Joystick-controlled laser pointer for presentations.

- Remote light dimmer.

- Virtual Etch-a-Sketch (draw using joystick on screen).

- Maze game using OLED + joystick.

- Security cam angle adjuster with servo and joystick.