11/08/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

DISTANCE MEASURING USING ULTRASONIC SENSOR, INDICATING USING LED AND BUZZER

DESCRIPTION:

This setup uses an HC-SR04 ultrasonic sensor to measure distance, with an LED and buzzer providing visual and audible alerts. A push button (configured with INPUT_PULLUP) is used to control or reset the system.

PIN CONNECTIONS:

| Component | Pin Connection | Arduino Pin | Notes |
| --- | --- | --- | --- |
| HC-SR04 VCC | VCC → 5V | — | Power supply for the sensor |
| HC-SR04 GND | GND → GND | — | Common ground |
| HC-SR04 Trig | Trig → D9 | D9 | Trigger pin for sending ultrasonic pulse |
| HC-SR04 Echo | Echo → D8 | D8 | Receives echo signal |
| LED Positive | Positive leg → Resistor → D3 | D3 | Use 220Ω–330Ω resistor in series |
| LED Negative | Negative leg → GND | — | Common ground |
| Buzzer Positive | Positive → D4 | D4 | Active or passive buzzer |
| Buzzer Negative | Negative → GND | — | Common ground |
| Push Button Leg 1 | One leg → D2 | D2 | Configured with INPUT_PULLUP |
| Push Button Leg 2 | Other leg → GND | — | Common ground |

BOARD SETUP:

PROGRAM:

// Pin definitions

const int trigPin = 9;     // Ultrasonic Trigger

const int echoPin = 8;     // Ultrasonic Echo

const int ledPin = 3;      // LED output

const int buzzerPin = 4;   // Buzzer output

const int buttonPin = 2;   // Push button input

// Variables

long duration;

float distance;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP); // Button with internal pull-up resistor

Serial.begin(9600);

}

void loop() {

// --- Ultrasonic Sensor Measurement ---

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.0343 / 2; // Convert to cm

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

// --- Button Press Action ---

if (digitalRead(buttonPin) == LOW) {  // Button pressed

digitalWrite(ledPin, HIGH);         // Turn LED on

digitalWrite(buzzerPin, HIGH);      // Turn buzzer on

delay(200);

digitalWrite(ledPin, LOW);          // Turn LED off

digitalWrite(buzzerPin, LOW);       // Turn buzzer off

delay(200);

}

}

OUTPUT: