30/07/2025

CUBICLE 5:

TEAM A:

SARMATHA S [23ECB28]

SUBAGARNIYA M [23ECB46]

SYED ALI FATHIMA A [23ECB50]

Magnetic Security Alarm using HW-497 & Buzzer

DESCRIPTION:

This project is a magnetic security alarm using an Arduino and HW-497 Hall Effect sensor. When a magnet moves away from the sensor (like when a door opens), the system triggers an LED and buzzer as an alert. It’s a simple and effective setup for basic security or door monitoring.

COMPONENTS:

| Component | Use |
| --- | --- |
| Arduino UNO/Nano | The brain |
| HW-497 Sensor | Detects magnetic field |
| LED | Visual alert |
| Buzzer | Sound alert |
| Jumper Wires | Connections |
| Resistor (220Ω) | For LED safety |

CONNECTIONS:

| HW-497 Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| OUT | D2 |

| LED Pin | Arduino Pin |
| --- | --- |
| + (Anode) | D12 |
| - (Cathode) | GND via 220Ω resistor |

| Buzzer Pin | Arduino Pin |
| --- | --- |
| + | D13 |
| - | GND |

PROGRAM:

int hallSensor = 2;     // HW-497 output pin

int ledPin = 12;        // LED pin

int buzzerPin = 13;     // Buzzer pin

void setup() {

pinMode(hallSensor, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int sensorValue = digitalRead(hallSensor);

if (sensorValue == HIGH) {

// Magnet is away —> alert ON

digitalWrite(ledPin, HIGH);

digitalWrite(buzzerPin, HIGH);

Serial.println("⚠️ Alert: Door open or Magnet missing!");

} else {

// Magnet is near —> alert OFF

digitalWrite(ledPin, LOW);

digitalWrite(buzzerPin, LOW);

Serial.println("✅ Safe: Magnet detected.");

}

delay(200);

}

OUTPUT:

4 Relay module:

It is an electronic module that contains four relays — electromagnetic switches used to control high-voltage or high

PROJECT:

1. Home Automation System

- Description: Control lights, fans, and appliances remotely using a microcontroller (like Arduino or ESP32) and a 4-relay module.

- Features: Wi-Fi/Bluetooth control, scheduled ON/OFF, voice commands (via Alexa or Google Assistant).

2. Smart Irrigation System

- Description: Automatically turn ON/OFF water pumps for different fields/zones based on soil moisture.

- Additional Components: Soil moisture sensor, water pump, ESP32, and mobile app (like Blynk).

3. Industrial Equipment Controller

- Description: Control 4 different machines or devices in a factory setup using a single embedded system.

- Features: Relay control through touchscreen or physical buttons.

4. IoT Energy Saver

- Description: Automatically turn off unused appliances (AC, heaters, lights) when no presence is detected.

- Sensors Used: PIR sensor, RTC module, or IR sensor.

5. Multi-Device Timer Switch

- Description: Turn ON/OFF 4 different appliances based on preset timer values.

- Features: LCD display, push buttons for setting time.

6. Remote-Controlled Power Distribution

- Description: Control power supply to 4 outputs using RF or IR remote.

- Modules: IR receiver or RF module with microcontroller.

7. Automatic Emergency Power Switch

- Description: Switch between main power and backup power (like inverter) for 4 devices.

- Logic: Relay-based switching with voltage sensors.

MAIN PURPOSE OF 4 RELAY MODULE:

- Act as an electronic switch to turn devices ON or OFF

- Control 4 different AC or DC loads independently

- Provide isolation between the control circuit and high-power devices using optocouplers

- Enable automation in home, industrial, or agricultural applications

Common Devices You Can Control:

- Lights

- Fans

- Motors

- Pumps

- Home appliances (TV, fridge, etc.)

- Power sockets

How It Works:

A microcontroller sends a LOW or HIGH signal → Relay coil gets activated → Switch inside the relay connects or disconnects the power to the device.